Assignment 9

Name-Aditya Agrawal

Enroll no-S24CSEU1321

- A shift register is a sequential logic circuit that stores and shifts binary data (1s and 0s) in a linear fashion. It consists of a cascade of flip-flops (usually D-type), where the output of one flip-flop is connected to the input of the next. Data can be shifted left or right, depending on the design.

Types of Shift Registers:

- Serial-In Serial-Out (SISO) Shift Register

- Serial-In Parallel-Out (SIPO) Shift Register

- Parallel-In Serial-Out (PISO) Shift Register

- Parallel-In Parallel-Out (PIPO) Shift Register

- Bidirectional Shift Register

## Applications of Shift Registers:

## Data Storage & Transfer, Time Delay Circuits, Arithmetic Operations etc.

- 2_verilog:

## //Aman Chaudhary

## //S24CSEU1348

## module Register_4bit (

## input wire clk,         // Clock signal

## input wire reset,       // Asynchronous reset (active high)

## input wire load,        // Load control signal (1 = load, 0 = hold)

## input wire [3:0] d,     // 4-bit input data

## output reg [3:0] q      // 4-bit output (registered value)

## );

## // Register behavior

## always @(posedge clk or posedge reset) begin

## if (reset) begin

## q <= 4'b0000;      // Reset to 0 (asynchronous)

## end

## else if (load) begin

## q <= d;            // Load new data on clock edge if load=1

## end

## // Else, retain previous value (implicit)

## end

## endmodule

## 2_testbench:

## `timescale 1ns / 1ps

## module Register_4bit_tb;

## // Inputs

## reg clk;

## reg reset;

## reg load;

## reg [3:0] d;

## // Outputs

## wire [3:0] q;

## // Instantiate the Unit Under Test (UUT)

## Register_4bit uut (

## .clk(clk),

## .reset(reset),

## .load(load),

## .d(d),

## .q(q)

## );

## // Clock generation (10ns period)

## initial begin

## clk = 0;

## forever #5 clk = ~clk;  // Toggle every 5ns (100MHz clock)

## end

## // Stimulus generation

## initial begin

## // Initialize Inputs

## reset = 1;

## load = 0;

## d = 4'b0000;

## // Dump waveform (for GTKWave/Vivado/ModelSim)

## $dumpfile("register_4bit_wave.vcd");

## $dumpvars(0, Register_4bit_tb);  // Dump all variables

## // Release reset after 10ns

## #10 reset = 0;

## // Test case 1: Load data (1010)

## d = 4'b1010;

## load = 1;

## #10 load = 0;

## // Test case 2: Change input but don't load (q should not change)

## d = 4'b1111;

## #10;

## // Test case 3: Load new data (1111)

## load = 1;

## #10 load = 0;

## // Test case 4: Reset again

## reset = 1;

## #10 reset = 0;

## // Test case 5: Load new data (0101)

## d = 4'b0101;

## load = 1;

## #10 load = 0;

## // End simulation

## #20 $finish;

## end

## // Monitor changes (optional)

## initial begin

## $monitor("Time = %0t ns | clk=%b reset=%b load=%b d=%4b q=%4b",

## $time, clk, reset, load, d, q);

## end

## endmodule

## 2_waveform:

- 3_verilog:

## //Aman Chaudhary

## //S24CSEU1348

## module ShiftRegister_8bit (

## input wire clk,         // Clock signal

## input wire reset,        // Asynchronous reset (active high)

## input wire ser_in,       // Serial input (shifted in LSB-first)

## output reg [7:0] q       // 8-bit parallel output

## );

## // Shift register behavior

## always @(posedge clk or posedge reset) begin

## if (reset) begin

## q <= 8'b00000000;    // Reset all bits to 0

## end

## else begin

## q <= {ser_in, q[7:1]};  // Right shift (MSB gets ser_in, others shift right)

## end

## end

## endmodule

## 3_testbench:

## `timescale 1ns / 1ps

## module ShiftRegister_8bit_tb;

## // Inputs

## reg clk;

## reg reset;

## reg ser_in;

## // Outputs

## wire [7:0] q;

## // Instantiate the shift register

## ShiftRegister_8bit uut (

## .clk(clk),

## .reset(reset),

## .ser_in(ser_in),

## .q(q)

## );

## // Clock generation (10ns period)

## initial begin

## clk = 0;

## forever #5 clk = ~clk;  // Toggle every 5ns (100MHz clock)

## end

## // Stimulus generation

## initial begin

## // Initialize

## reset = 1;

## ser_in = 0;

## // Dump waveform (for GTKWave/Vivado/ModelSim)

## $dumpfile("shift_register_8bit_wave.vcd");

## $dumpvars(0, ShiftRegister_8bit_tb);

## // Release reset after 10ns

## #10 reset = 0;

## // Shift in data: 11010010 (LSB first)

## ser_in = 0; #10; // Bit 0

## ser_in = 1; #10; // Bit 1

## ser_in = 0; #10; // Bit 2

## ser_in = 0; #10; // Bit 3

## ser_in = 1; #10; // Bit 4

## ser_in = 0; #10; // Bit 5

## ser_in = 1; #10; // Bit 6

## ser_in = 1; #10; // Bit 7

## // Check output (should be 11010010)

## #10;

## // Reset and test again

## reset = 1;

## #10 reset = 0;

## ser_in = 1;

## #80;  // Shift in 1s (result: 11111111)

## // End simulation

## #10 $finish;

## end

## // Monitor changes

## initial begin

## $monitor("Time = %0t ns | clk=%b reset=%b ser_in=%b q=%8b",

## $time, clk, reset, ser_in, q);

## end

## endmodule

## 3_waveform: